IPAd Library

Developer Reference Manual

Version: 1.0.0

Standard: GSMA SGP.32 — IoT Profile Assistant Device

Platform: Qualcomm SA525 · Telematics SDK

Language: C++17 / C API

Date: May 31, 2026

Status: Released

# Table of Contents

1. Introduction

2. Architecture Overview

3. Build System

4. Public API

4.1  Type Definitions

4.2  Status Codes

4.3  Enumerations

4.4  Data Structures

4.5  Lifecycle

4.6  eUICC Identity

4.7  Profile Management

4.8  Event Subsystem

4.9  Logging

4.10 Diagnostics

4.11 Utility Functions

5. Log Sink Module (ipad_log.h)

6. Mock Backend

7. Python Port

8. Error Handling

9. Thread Safety

10. Performance & KPIs

11. Typical Usage Examples

12. Changelog

# 1. Introduction

The IPAd library (IoT Profile Assistant Device) is a C/C++ abstraction layer over the Qualcomm Telematics SDK (TelSDK) for the SA525 LTE/5G modem. It implements the GSMA SGP.32 specification, enabling IoT devices to manage eSIM profiles over-the-air without requiring a physical SIM card slot.

The library exposes a pure-C API (extern "C" linkage) so it can be consumed from C, C++, Python (via the bundled Python port), or any language with a foreign function interface.

## Key features

Full SGP.32 profile lifecycle: download, enable, disable, delete.

Synchronous and asynchronous profile download (ipad_profile_download / _async).

Event subscription system with bitmask filtering.

Configurable structured logging: NONE / INFO / DEBUG, serial or TCP sink.

Built-in diagnostics export (JSON snapshot).

Hardware-free mock backend for CI testing (IPAD_MOCK_TELSDK=ON).

Python 3.9+ port with identical semantics.

Automatic PDF KPI report generated at every cmake build.

# 2. Architecture Overview

The library is structured in three layers:

| Layer | Files | Description |
| --- | --- | --- |
| Public API | include/ipad/ipad.h include/ipad/ipad_log.h | Stable C interface. Never include internal headers. |
| Implementation | src/ipad.cpp src/ipad_log.cpp | C++17 implementation. Uses TelSDK or mock backend. |
| Backend | include/ipad/ipad_telsdk_backend.h tests/mocks/mock_telsdk.h | Compile-time backend selection via IPAD_MOCK_TELSDK. |

## Dependency graph

Application
  └── libipad.so  (ipad.h / ipad_log.h)
        ├── libtelux_tel.so     (real hardware)
        ├── libtelux_common.so  (real hardware)
        └── mock_telsdk.h       (IPAD_MOCK_TELSDK=ON)

## Internal context (ipad_ctx_s)

Each ipad_handle_t points to a heap-allocated ipad_ctx_s containing:

| Field | Type | Purpose |
| --- | --- | --- |
| cfg | ipad_config_t | Runtime configuration copy |
| sim_mgr_ | shared_ptr<ISimProfileManager> | TelSDK profile manager |
| card_mgr_ | shared_ptr<ICardManager> | TelSDK card manager |
| event_queue_ | std::queue<ipad_event_t> | Pending events (thread-safe) |
| subscriptions_ | std::map<uint32_t, Subscription> | Active event listeners |
| log_cb_ | ipad_log_cb_t | User log callback |
| log_level_ | ipad_loglevel_t | Current log verbosity |
| mtx_ | std::mutex | Protects event queue and subs |
| cv_ | std::condition_variable | Wakes pump thread |

# 3. Build System

The project uses CMake 3.16+. The build options are:

| CMake Option | Default | Description |
| --- | --- | --- |
| IPAD_MOCK_TELSDK | OFF | Use in-process mock instead of real TelSDK. Required when SA525 hardware is absent. |
| BUILD_APP | ON | Build the ncurses TUI demo application (ipad_app). |
| BUILD_TESTS | ON | Build the GTest test suite. |
| BUILD_COVERAGE | OFF | Instrument with gcov for coverage measurement. |

## Quick start

# Clone and build (mock mode, no hardware needed)
git clone https://github.com/your-org/ipad_lib_sgp32.git
cd ipad_lib_sgp32/ipad_lib
cmake -B build -DIPAD_MOCK_TELSDK=ON -DCMAKE_BUILD_TYPE=Release
cmake --build build -j$(nproc)

# Run tests
ctest --test-dir build --output-on-failure

# Real hardware (SA525)
export TELSDK_ROOT=/opt/qualcomm/telsdk
cmake -B build_hw -DCMAKE_BUILD_TYPE=Release
cmake --build build_hw -j$(nproc)

## Required dependencies

| Package | Version | Purpose | apt package |
| --- | --- | --- | --- |
| GTest | ≥ 1.11 | Unit/integration tests | libgtest-dev |
| nlohmann/json | ≥ 3.10 | Test vector runner | nlohmann-json3-dev |
| ncurses | any | TUI demo application | libncurses-dev |
| reportlab | ≥ 4.0 | PDF KPI report generation | pip install reportlab |

# 4. Public API

Include only the public header in application code:

#include "ipad/ipad.h"
#include "ipad/ipad_log.h"   // optional log sink

## 4.1  Type Definitions

| Type | Underlying type | Description |
| --- | --- | --- |
| ipad_status_t | int32_t | Return code for all API functions |
| ipad_handle_t | struct ipad_ctx_s * | Opaque library instance handle |
| ipad_eid_t | uint8_t[16] | 32 BCD digits packed into 16 bytes |
| ipad_iccid_t | uint8_t[10] | 20 BCD digits packed into 10 bytes |
| ipad_sub_id_t | uint32_t | Event subscription identifier |
| ipad_event_cb_t | function pointer | Event callback type |
| ipad_callback_t | function pointer | Async operation callback type |
| ipad_log_cb_t | function pointer | Log message callback type |

## 4.2  Status Codes

| Constant | Value | Meaning |
| --- | --- | --- |
| IPAD_OK | 0 | Operation completed successfully |
| IPAD_PENDING | 1 | Asynchronous operation started |
| IPAD_ERR_INVALID_PARAM | -1 | NULL pointer or out-of-range argument |
| IPAD_ERR_NOT_INITIALIZED | -2 | ipad_init() not called |
| IPAD_ERR_HAL_FAILURE | -3 | Hardware / driver error |
| IPAD_ERR_TRANSPORT | -4 | Network transport failure |
| IPAD_ERR_TLS_HANDSHAKE | -5 | TLS negotiation failed |
| IPAD_ERR_EIM_REJECTED | -6 | eIM server rejected the request |
| IPAD_ERR_PROFILE_NOT_FOUND | -7 | ICCID does not match any profile |
| IPAD_ERR_PROFILE_ACTIVE | -8 | Cannot delete an active profile |
| IPAD_ERR_NO_MEMORY | -9 | Memory allocation failed |
| IPAD_ERR_CRYPTO | -10 | Cryptographic operation failed |
| IPAD_ERR_TIMEOUT | -11 | Operation exceeded timeout |
| IPAD_ERR_BUSY | -12 | Resource locked by another operation |
| IPAD_ERR_NOT_SUPPORTED | -13 | Feature not available on this platform |
| IPAD_ERR_TELSDK | -14 | TelSDK backend returned an error |
| IPAD_ERR_INTERNAL | -99 | Unexpected internal error |

## 4.3  Enumerations

### ipad_transport_t

| Value | Integer | Description |
| --- | --- | --- |
| IPAD_TRANSPORT_TCP | 0 | Standard TCP/IP (default) |
| IPAD_TRANSPORT_COAP | 1 | CoAP over UDP |
| IPAD_TRANSPORT_MQTT | 2 | MQTT over TCP |
| IPAD_TRANSPORT_SMS | 3 | SMS bearer (fallback) |

### ipad_loglevel_t

| Value | Integer | Description |
| --- | --- | --- |
| IPAD_LOG_NONE | 0 | No log output |
| IPAD_LOG_ERROR | 1 | Errors only |
| IPAD_LOG_WARN | 2 | Warnings and errors |
| IPAD_LOG_INFO | 3 | Informational messages |
| IPAD_LOG_DEBUG | 4 | Full debug trace |

### ipad_profile_state_t

| Value | Integer | Description |
| --- | --- | --- |
| IPAD_PROFILE_ENABLED | 0 | Profile is active (in use) |
| IPAD_PROFILE_DISABLED | 1 | Profile installed but inactive |
| IPAD_PROFILE_PENDING_INSTALL | 2 | Download in progress |
| IPAD_PROFILE_PENDING_ENABLE | 3 | Waiting for activation |
| IPAD_PROFILE_ERROR | 4 | Profile in error state |

### ipad_event_type_t  (bitmask)

| Value | Bit | Description |
| --- | --- | --- |
| IPAD_EVT_PROFILE_INSTALLED | 0 | New profile successfully installed |
| IPAD_EVT_PROFILE_ENABLED | 1 | Profile was activated |
| IPAD_EVT_PROFILE_DISABLED | 2 | Profile was deactivated |
| IPAD_EVT_PROFILE_DELETED | 3 | Profile was permanently removed |
| IPAD_EVT_EIM_CONNECTED | 4 | eIM server connection established |
| IPAD_EVT_EIM_DISCONNECTED | 5 | eIM server connection lost |
| IPAD_EVT_NETWORK_ATTACHED | 6 | LTE/5G network attachment |
| IPAD_EVT_NETWORK_DETACHED | 7 | Network detachment |
| IPAD_EVT_DOWNLOAD_PROGRESS | 8 | Download percentage update |
| IPAD_EVT_ERROR | 9 | Unrecoverable error occurred |
| IPAD_EVT_ALL | 0xFFFFFFFF | Subscribe to all event types |

## 4.4  Data Structures

### ipad_config_t

Library initialisation parameters

| Field | Type | Description |
| --- | --- | --- |
| eim_url | const char * | eIM endpoint URL, e.g. "coaps://eim.host:5684" |
| eim_port | uint16_t | eIM TCP/UDP port (0 = use URL default) |
| transport | ipad_transport_t | Transport protocol |
| tls | ipad_tls_cfg_t | TLS certificate paths and options |
| timeout_ms | uint32_t | Default operation timeout (ms). 0 → 30 000 |
| retry_count | uint8_t | Automatic retry count. 0 → 3 |
| log_level | ipad_loglevel_t | Initial log verbosity |
| slot_id | uint32_t | SA525 SIM slot index (0-based) |
| user_ctx | void * | Opaque pointer passed to callbacks |

### ipad_dl_params_t

Parameters for profile download

| Field | Type | Description |
| --- | --- | --- |
| activation_code | const char * | LPA activation code: "LPA:1$host$matchid" |
| confirmation_code | const char * | Confirmation code (optional, may be NULL) |
| auto_enable | bool | Enable the profile immediately after install |
| timeout_ms | uint32_t | Override timeout. 0 = use ipad_config_t.timeout_ms |

### ipad_profile_t

Installed eSIM profile descriptor

| Field | Type | Description |
| --- | --- | --- |
| iccid | ipad_iccid_t | 20 BCD digits packed as 10 bytes |
| state | ipad_profile_state_t | Current state |
| nickname | char[64] | User-editable label |
| provider | char[64] | Operator / MNO name |
| profile_name | char[64] | Profile product name |
| is_test_profile | bool | True for test/bootstrap profiles |
| slot_id | uint32_t | Physical SIM slot index |

### ipad_euicc_info_t

eUICC identity and capabilities

| Field | Type | Description |
| --- | --- | --- |
| eid | ipad_eid_t | 32 BCD digits packed as 16 bytes |
| eid_str | char[33] | Null-terminated uppercase hex string |
| os_version | char[64] | eUICC OS version string |
| profile_capacity | uint32_t | Maximum number of simultaneous profiles |
| free_mem_kb | uint32_t | Available eUICC non-volatile memory (KB) |

### ipad_event_t

Event payload delivered to subscribers

| Field | Type | Description |
| --- | --- | --- |
| type | ipad_event_type_t | Event type bitmask value |
| iccid | ipad_iccid_t | Relevant profile ICCID (zero if N/A) |
| status | ipad_status_t | IPAD_OK or error code for the event |
| progress_pct | uint8_t | 0–100 for IPAD_EVT_DOWNLOAD_PROGRESS |
| detail | char[128] | Human-readable description |

### ipad_tls_cfg_t

TLS transport configuration

| Field | Type | Description |
| --- | --- | --- |
| ca_cert_path | const char * | Path to PEM CA bundle |
| client_cert_path | const char * | Path to client certificate (PEM) |
| client_key_path | const char * | Path to client private key (PEM) |
| verify_peer | bool | Enable server certificate verification |
| tls_version_min | uint16_t | 0x0303 = TLS 1.2, 0x0304 = TLS 1.3 |

### ipad_version_t

Library and TelSDK version information

| Field | Type | Description |
| --- | --- | --- |
| major | uint16_t | Library major version |
| minor | uint16_t | Library minor version |
| patch | uint16_t | Library patch version |
| sgp32_revision | const char * | Supported SGP.32 specification revision |
| telsdk_version | const char * | TelSDK version linked against |

## 4.5  Lifecycle

### ipad_init

ipad_status_t ipad_init(const ipad_config_t *cfg, ipad_handle_t *hdl_out);

Initialise the library and connect to TelSDK. Must be called before any other ipad_* function. Blocks until the TelSDK subsystem reports ready or cfg->timeout_ms elapses.

| Parameter | Description |
| --- | --- |
| cfg | Non-NULL pointer to configuration. Copied internally. |
| hdl_out | Receives the new handle on success. |

Returns: IPAD_OK, IPAD_ERR_INVALID_PARAM, IPAD_ERR_TELSDK, IPAD_ERR_TIMEOUT

### ipad_deinit

ipad_status_t ipad_deinit(ipad_handle_t hdl);

Release all resources, disconnect from TelSDK, and free the handle. After this call hdl is invalid.

| Parameter | Description |
| --- | --- |
| hdl | Handle obtained from ipad_init(). |

Returns: IPAD_OK, IPAD_ERR_INVALID_PARAM

### ipad_configure

ipad_status_t ipad_configure(ipad_handle_t hdl, const ipad_config_t *cfg);

Update transport, log level, and retry policy at runtime. Does not restart TelSDK connections.

| Parameter | Description |
| --- | --- |
| hdl | Valid handle. |
| cfg | New configuration. |

Returns: IPAD_OK, IPAD_ERR_INVALID_PARAM

### ipad_get_version

ipad_status_t ipad_get_version(ipad_version_t *ver_out);

Retrieve library and TelSDK version. Does not require a valid handle.

| Parameter | Description |
| --- | --- |
| ver_out | Caller-allocated version structure. |

Returns: IPAD_OK, IPAD_ERR_INVALID_PARAM

### ipad_selftest

ipad_status_t ipad_selftest(ipad_handle_t hdl, uint32_t *result_bitmask);

Run internal self-tests (HAL, transport, crypto). Each bit in the output corresponds to a sub-system: bit 0 = HAL, bit 1 = TRANSPORT, bit 2 = CRYPTO.

| Parameter | Description |
| --- | --- |
| hdl | Valid handle. |
| result_bitmask | Receives the bitfield of passed tests. |

Returns: IPAD_OK, IPAD_ERR_INVALID_PARAM

## 4.6  eUICC Identity

### ipad_get_eid

ipad_status_t ipad_get_eid(ipad_handle_t hdl, ipad_eid_t eid_out);

Read the EID (Equipment Identifier) of the embedded SIM. eid_out must point to a caller-allocated buffer of at least IPAD_EID_LEN (16) bytes.

### ipad_get_euicc_info

ipad_status_t ipad_get_euicc_info(ipad_handle_t hdl, ipad_euicc_info_t *info_out);

Retrieve extended eUICC information including EID string, OS version, profile capacity, and available non-volatile memory.

## 4.7  Profile Management

### ipad_profile_list

ipad_status_t ipad_profile_list(
    ipad_handle_t  hdl,
    ipad_profile_t *list_out,
    uint8_t        *count_inout);

Retrieve all profiles installed on the eUICC.

Two-pass query pattern: call first with list_out = NULL to obtain the count, allocate the array, then call again with the array.

count_inout: on entry, the capacity of list_out. On exit, the actual number of profiles returned.

### ipad_profile_get

ipad_status_t ipad_profile_get(
    ipad_handle_t    hdl,
    const ipad_iccid_t iccid,
    ipad_profile_t  *profile_out);

Retrieve a single profile by ICCID. Returns IPAD_ERR_PROFILE_NOT_FOUND if no match.

### ipad_profile_download

ipad_status_t ipad_profile_download(
    ipad_handle_t          hdl,
    const ipad_dl_params_t *params,
    ipad_iccid_t            iccid_out);

Synchronous profile download from an SM-DP+ server via TelSDK addProfile. Blocks until the download completes or fails. On success, iccid_out receives the ICCID of the newly installed profile. iccid_out may be NULL if the caller does not need the ICCID.

### ipad_profile_download_async

ipad_status_t ipad_profile_download_async(
    ipad_handle_t           hdl,
    const ipad_dl_params_t *params,
    ipad_callback_t         cb,
    void                   *cb_ctx);

Asynchronous variant. Returns IPAD_PENDING immediately and invokes cb when the operation completes. The result pointer passed to cb is an ipad_iccid_t * on success, NULL on failure.

### ipad_profile_enable

ipad_status_t ipad_profile_enable(ipad_handle_t hdl, const ipad_iccid_t iccid);

Activate a profile. Maps to TelSDK setProfile(ENABLE). The modem may reset briefly to apply the new profile.

### ipad_profile_disable

ipad_status_t ipad_profile_disable(ipad_handle_t hdl, const ipad_iccid_t iccid);

Deactivate a profile. The profile remains installed and can be re-enabled.

### ipad_profile_delete

ipad_status_t ipad_profile_delete(ipad_handle_t hdl, const ipad_iccid_t iccid);

Permanently delete a profile. Returns IPAD_ERR_PROFILE_ACTIVE if the profile is currently enabled. Disable first before deleting.

### ipad_profile_set_nickname

ipad_status_t ipad_profile_set_nickname(
    ipad_handle_t      hdl,
    const ipad_iccid_t iccid,
    const char        *nickname);

Update the user-visible nickname for a profile. Maximum length: IPAD_STR_MAX-1 (63 chars).

### ipad_profile_get_state

ipad_status_t ipad_profile_get_state(
    ipad_handle_t         hdl,
    const ipad_iccid_t    iccid,
    ipad_profile_state_t *state_out);

Query the current state of a profile without fetching full details.

## 4.8  Event Subsystem

The event subsystem allows the application to receive asynchronous notifications about profile state changes, network events, and download progress. Multiple subscriptions can coexist; each is filtered by a bitmask.

### ipad_event_subscribe

ipad_sub_id_t ipad_event_subscribe(
    ipad_handle_t   hdl,
    uint32_t        event_mask,
    ipad_event_cb_t cb,
    void           *user_ctx);

Register an event listener. event_mask is a bitwise OR of ipad_event_type_t values. Use IPAD_EVT_ALL to receive all events. Returns the subscription ID (> 0) on success, 0 on failure. The callback is invoked from the library's internal worker thread.

WARNING: The callback is invoked from a background thread. Protect shared data with a mutex.

### ipad_event_unsubscribe

ipad_status_t ipad_event_unsubscribe(ipad_handle_t hdl, ipad_sub_id_t sub_id);

Cancel a subscription. After this call no further events are delivered to that callback.

### ipad_event_pump

ipad_status_t ipad_event_pump(ipad_handle_t hdl, uint32_t timeout_ms);

Process pending events for up to timeout_ms milliseconds. Useful in single-threaded environments that cannot receive callbacks from a background thread. In multi-threaded applications this function is optional.

## 4.9  Logging

The library emits structured log messages via a registered callback. The log_level set at init time acts as the global filter; the callback receives only messages at or below that level.

### ipad_log_set_handler

ipad_status_t ipad_log_set_handler(
    ipad_handle_t hdl,
    ipad_log_cb_t  cb,
    void          *user_ctx);

Install a log callback. Pass cb = NULL to disable logging. The callback signature is: void cb(ipad_loglevel_t, const char *module, const char *message, void *ctx).

### ipad_log_set_level

ipad_status_t ipad_log_set_level(ipad_handle_t hdl, ipad_loglevel_t level);

Update the log verbosity at runtime without replacing the callback.

## 4.10  Diagnostics

### ipad_diag_export_json

ipad_status_t ipad_diag_export_json(
    ipad_handle_t hdl,
    char         *buf,
    size_t        buf_len);

Export a JSON diagnostic snapshot into buf. The snapshot includes library version, EID, profile list with states, current log level, and TelSDK status. If buf is NULL, returns the required buffer size in buf_len.

// Example output:
{
  "library_version": "1.0.0",
  "eid": "89049032123456789012345678901234",
  "profile_count": 2,
  "profiles": [
    { "iccid": "8901234...", "state": "ENABLED", "nickname": "Work" },
    { "iccid": "8901235...", "state": "DISABLED", "nickname": "" }
  ]
}

## 4.11  Utility Functions

| Function | Signature | Description |
| --- | --- | --- |
| ipad_strerror | const char *ipad_strerror(ipad_status_t) | Return a human-readable string for a status code. Thread-safe, never NULL. |
| ipad_iccid_to_str | void ipad_iccid_to_str(const ipad_iccid_t, char *str_out) | Convert 10-byte binary ICCID to 20-char uppercase hex + NUL. str_out must be ≥ 21 bytes. |
| ipad_eid_to_str | void ipad_eid_to_str(const ipad_eid_t, char *str_out) | Convert 16-byte binary EID to 32-char uppercase hex + NUL. str_out must be ≥ 33 bytes. |

# 5. Log Sink Module  (ipad_log.h)

The log sink module provides ready-made transport back-ends for the library log stream. It wraps ipad_log_set_handler() with automatic formatting (ISO-8601 timestamps, level tags) and two transport options: serial port and TCP client.

## User-facing log levels

| Level | Value | Format | Internal mapping |
| --- | --- | --- | --- |
| IPAD_LOGLVL_NONE | 0 | No output | IPAD_LOG_NONE |
| IPAD_LOGLVL_SIMPLE | 1 | [YYYY-MM-DD HH:MM:SS] [LEVEL] [module] message | IPAD_LOG_INFO |
| IPAD_LOGLVL_DETAIL | 3 | [YYYY-MM-DD HH:MM:SS.mmm] [LEVEL] [module] message | IPAD_LOG_DEBUG |

## Serial sink

ipad_status_t ipad_log_sink_serial(
    ipad_log_sink_t *sink,
    const char      *device,    // e.g. "/dev/ttyUSB0"
    uint32_t         baudrate); // e.g. 115200

Opens the tty device in raw 8N1 mode with the specified baud rate. Returns IPAD_ERR_HAL_FAILURE if the device cannot be opened. Common baud rates: 9600, 19200, 38400, 57600, 115200, 230400, 460800, 921600.

## TCP sink

ipad_status_t ipad_log_sink_tcp(
    ipad_log_sink_t *sink,
    const char      *host,  // IPv4 or hostname
    uint16_t         port); // 1-65535

Connects immediately to host:port. TCP_NODELAY is set so each log line is delivered without buffering. If the connection is lost, subsequent writes are silently dropped. Returns IPAD_ERR_TRANSPORT on connection failure.

NOTE: Use `nc -l <port>` on the receiving host to view log output in a terminal.

## Attach / Detach

ipad_status_t ipad_log_attach(ipad_handle_t, ipad_log_sink_t *, ipad_user_loglvl_t);
ipad_status_t ipad_log_detach(ipad_handle_t, ipad_log_sink_t *);
void          ipad_log_sink_close(ipad_log_sink_t *);

ipad_log_attach() installs the sink as the active log handler for the handle. Only one sink is active at a time; calling attach again replaces the previous one. ipad_log_detach() restores silent mode (IPAD_LOG_NONE, NULL handler). ipad_log_sink_close() closes the underlying file descriptor.

# 6. Mock Backend

When compiled with -DIPAD_MOCK_TELSDK=ON, the library links against an in-process simulation of TelSDK instead of the real Qualcomm libraries. This enables unit testing on any Linux machine without SA525 hardware.

## Mock configuration knobs

| Field (MockConfig) | Type | Default | Effect |
| --- | --- | --- | --- |
| subsystem_ready | bool | true | If false, ipad_init() returns ERR_TELSDK |
| subsystem_ready_timeout | bool | false | If true, ipad_init() returns ERR_TIMEOUT |
| fail_add_profile | bool | false | Download returns ERR_TELSDK |
| fail_add_profile_status | bool | false | Download returns ERR_EIM_REJECTED |
| fail_delete_profile | bool | false | Delete returns ERR_TELSDK |
| fail_set_profile | bool | false | Enable/disable return ERR_TELSDK |
| fail_get_profiles | bool | false | Profile list returns ERR_TELSDK |
| require_user_consent | bool | false | Simulates user consent flow |
| download_error_at_end | bool | false | Download fails at completion step |
| download_delay_ms | int | 50 | Simulated download duration (ms) |

# 7. Python Port

A full Python 3.9+ port is provided in ipad_python/. It has identical semantics to the C library and shares the same mock backend concept, making it suitable for integration tooling, test automation, and scripted deployments.

## Package structure

| Module | Description |
| --- | --- |
| ipad.types | Enums, dataclasses, IpadError, utility functions |
| ipad.backend | Abstract IpadBackend interface |
| ipad.mock_backend | MockBackend with MockConfig (mirrors C++ mock) |
| ipad.handle | IpadHandle — main entry point, context manager |
| ipad.log_sink | log_sink_serial(), log_sink_tcp(), IpadLogSink |

## Quick start (Python)

from ipad import IpadHandle, IpadConfig, IpadDlParams, IpadLogLevel, MockBackend
from ipad.log_sink import log_sink_tcp, IpadUserLogLevel

cfg = IpadConfig(timeout_ms=5000, log_level=IpadLogLevel.DEBUG)
with IpadHandle(cfg, backend=MockBackend()) as hdl:
    # Attach TCP log sink
    sink = log_sink_tcp('127.0.0.1', 9000)
    sink.attach(hdl, IpadUserLogLevel.SIMPLE)

    # Download a profile
    iccid = hdl.profile_download(
        IpadDlParams(activation_code='LPA:1$smdp.example.com$MATCH01')
    )
    print('Installed:', hdl.iccid_to_str(iccid))

    sink.detach(hdl)
    sink.close()

## Running tests

cd ipad_lib_sgp32/ipad_python
pip install pytest
pytest tests/ -v           # 54 tests, all pass

# 8. Error Handling

All API functions return ipad_status_t. A return value of IPAD_OK (0) indicates success; any negative value indicates an error. IPAD_PENDING (1) is returned only by ipad_profile_download_async() to indicate the operation was started.

ipad_status_t rc = ipad_profile_enable(hdl, iccid);
if (rc != IPAD_OK) {
    fprintf(stderr, "enable failed: %s\n", ipad_strerror(rc));
}

NOTE: In Python, non-OK status codes are raised as IpadError exceptions with a .status attribute of type IpadStatus.

# 9. Thread Safety

The library is designed for use from a single thread with callbacks arriving on a background worker thread. The following rules apply:

ipad_handle_t is not thread-safe for concurrent API calls. Serialize all calls from the application thread.

The event callback (ipad_event_cb_t) is invoked from the internal worker thread. Protect any shared data accessed from both the callback and the application thread.

ipad_log_set_handler() and ipad_log_set_level() are safe to call from any thread.

Multiple handles (from multiple ipad_init() calls) are fully independent and may be used concurrently from separate threads.

# 10. Performance & KPIs

The following measurements were obtained on a x86-64 Linux host (Release build, -O2, IPAD_MOCK_TELSDK=ON). A PDF KPI report is automatically regenerated after every cmake build and written to <build_dir>/ipad_kpi_report.pdf.

## Binary size

| Artefact | Debug symbols | Stripped |
| --- | --- | --- |
| libipad.so | 142 KB | 112 KB |
| ipad_app | 65 KB | 55 KB |
| Total | 207 KB | 167 KB |

## Library section breakdown

| Section | Size | Content |
| --- | --- | --- |
| .text | 50.5 KB | Machine code |
| .rodata | 3.8 KB | String literals, vtables, jump tables |
| .data | 1.1 KB | Initialised globals (incl. vtable pointers) |
| .bss | 80 B | Zero-initialised globals |

## RAM consumption (1 handle)

| Zone | Size | Notes |
| --- | --- | --- |
| ipad_ctx_s heap | ~5.3 KB | Allocated by ipad_init() |
| event queue (32 sl.) | 4.9 KB | Included in handle |
| eUICC info | 124 B | Included in handle |
| Worker thread stack | 64 KB | Linux default; tunable to 8 KB via pthread_attr |
| Static (.data + .bss) | ~3 KB | Loaded once, shared across handles |
| TOTAL (typical) | ~72 KB | 0.014 % of SA525 512 MB RAM |

NOTE: On the SA525 (512 MB RAM, 4 GB flash), the library occupies < 0.003 % of flash and < 0.02 % of RAM.

# 11. Typical Usage Examples

## 11.1  Initialise and download a profile (C)

#include "ipad/ipad.h"
#include "ipad/ipad_log.h"

int main(void) {
    /* 1. Configure */
    ipad_config_t cfg = {0};
    cfg.timeout_ms = 30000;
    cfg.log_level  = IPAD_LOG_INFO;

    /* 2. Init */
    ipad_handle_t hdl;
    if (ipad_init(&cfg, &hdl) != IPAD_OK) return 1;

    /* 3. Attach TCP log sink */
    ipad_log_sink_t sink;
    ipad_log_sink_tcp(&sink, "192.168.1.10", 9000);
    ipad_log_attach(hdl, &sink, IPAD_LOGLVL_SIMPLE);

    /* 4. Download */
    ipad_dl_params_t params = {0};
    params.activation_code = "LPA:1$smdp.example.com$MATCH01";
    params.auto_enable     = true;
    ipad_iccid_t iccid;
    ipad_status_t rc = ipad_profile_download(hdl, &params, iccid);

    /* 5. Print result */
    char str[21];
    ipad_iccid_to_str(iccid, str);
    printf("installed: %s  (%s)\n", str, ipad_strerror(rc));

    /* 6. Cleanup */
    ipad_log_detach(hdl, &sink);
    ipad_log_sink_close(&sink);
    ipad_deinit(hdl);
    return rc == IPAD_OK ? 0 : 1;
}

## 11.2  Event-driven profile monitoring (C)

static void on_event(ipad_handle_t hdl, const ipad_event_t *evt, void *ctx) {
    if (evt->type == IPAD_EVT_DOWNLOAD_PROGRESS)
        printf("progress: %d%%\n", evt->progress_pct);
    else if (evt->type == IPAD_EVT_PROFILE_INSTALLED)
        printf("profile installed\n");
}

ipad_sub_id_t sub = ipad_event_subscribe(
    hdl,
    IPAD_EVT_DOWNLOAD_PROGRESS | IPAD_EVT_PROFILE_INSTALLED,
    on_event, NULL);

/* ... trigger download ... */

ipad_event_unsubscribe(hdl, sub);

## 11.3  List and enable a profile (C++)

uint8_t count = 0;
ipad_profile_list(hdl, nullptr, &count);  // query count
std::vector<ipad_profile_t> profiles(count);
ipad_profile_list(hdl, profiles.data(), &count);

for (auto &p : profiles) {
    char s[21]; ipad_iccid_to_str(p.iccid, s);
    printf("%s  state=%d  nick=%s\n", s, p.state, p.nickname);
    if (p.state == IPAD_PROFILE_DISABLED)
        ipad_profile_enable(hdl, p.iccid);
}

# 12. Changelog

| Version | Date | Changes |
| --- | --- | --- |
| 1.0.0 | 2026-05-31 | Initial release. Full SGP.32 profile management, event system, configurable log sinks (serial + TCP), Python port, 70 tests (C), 54 tests (Python), automatic PDF KPI report generation. |

IPAd Library v1.0.0  ·  GSMA SGP.32  ·  Qualcomm SA525  ·  Generated May 31, 2026