IPAd Library

Requirements Traceability Matrix (RTM)

Product: IPAd — GSMA SGP.32 IoT Profile Assistant Device

Spec: GSMA SGP.32 v1.0

Backend: Qualcomm Telematics SDK (TelSDK) — SA525

Version: 1.0.0

Date: 2026-05-31

Status: 29/38 VERIFIED · 9 PARTIAL (config-only / indirect)

# Legend

| VERIFIED | Requirement implemented and covered by at least one automated test |
| --- | --- |
| PARTIAL | Requirement implemented; coverage limited to mock / unit level |
| NOT TESTED | Requirement implemented but no automated test exists |

# Coverage Summary

| Category | Total Reqs | Verified | Coverage |
| --- | --- | --- | --- |
| Lifecycle & Init | 5 | 5 | 100% |
| eUICC Identity | 2 | 2 | 100% |
| Profile Download | 5 | 5 | 100% |
| Profile Management | 8 | 8 | 100% |
| Transport | 4 | 0 | 0% (config only) |
| Security / TLS | 3 | 0 | 0% (config only) |
| Event System | 5 | 4 | 80% |
| Error Handling | 3 | 2 | 67% |
| Diagnostics | 3 | 3 | 100% |
| TOTAL | 38 | 29 | 76% |

# Full Requirements Traceability Matrix

| Req ID | Category | SGP.32 Ref | Requirement | Implementation | Tests | Status |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-001 | Lifecycle | SGP.32 §3.1 | The IPAd SHALL initialise a session with the eUICC management subsystem (TelSDK) before any profile operation. | src/ipad.cpp::ipad_init() | IpadInitTest.SuccessfulInit IpadInitTest.NullConfigReturnsInvalidParam IpadInitTest.InitNullCardManagerFails TV-INIT-001 TC_WF_01_FullLifecycle BenchInit.InitDeinitCycle | VERIFIED |
| REQ-002 | Lifecycle | SGP.32 §3.1 | The IPAd SHALL release all eUICC management resources on session termination. | src/ipad.cpp::ipad_deinit() | IpadInitTest.DeinitNullHandleIsNoop BenchInit.InitDeinitCycle | VERIFIED |
| REQ-003 | Lifecycle | SGP.32 §3.1 | The IPAd SHALL support runtime reconfiguration of timeout, log level, and retry count without restart. | src/ipad.cpp::ipad_configure() | IpadTest.ConfigureUpdatesSettings IpadTest.ConfigureNullParamsReturnsError | VERIFIED |
| REQ-004 | Lifecycle | SGP.32 §3.1 | The IPAd SHALL expose its library version and the supported SGP.32 specification revision. | src/ipad.cpp::ipad_get_version() ipad.h::ipad_version_t | IpadVersionTest.GetVersionPopulatesFields IpadVersionTest.GetVersionNullReturnsError | VERIFIED |
| REQ-005 | Lifecycle | SGP.32 §3.1 | The IPAd SHALL provide a built-in self-test covering HAL and TelSDK subsystems. | src/ipad.cpp::ipad_selftest() | IpadTest.SelfTestPasses | VERIFIED |
| REQ-010 | eUICC Identity | SGP.32 §3.2.1 | The IPAd SHALL retrieve the eUICC Identifier (EID) as a 32 BCD-digit (16-byte) value. | src/ipad.cpp::ipad_get_eid() ipad.h::IPAD_EID_LEN=16 | IpadTest.GetEidReturnsExpectedValue IpadTest.GetEidFailsGracefully IpadTest.GetEidNullHandleReturnsError TV-EID-001 TV-EID-002 BenchSuite.GetEid | VERIFIED |
| REQ-011 | eUICC Identity | SGP.32 §3.2.2 | The IPAd SHALL expose eUICC information including EID, OS version, profile capacity, and available memory. | src/ipad.cpp::ipad_get_euicc_info() ipad.h::ipad_euicc_info_t | BenchSuite.GetEuiccInfo TC_WF_07_DiagnosticAfterChanges (indirect) | VERIFIED |
| REQ-020 | Profile Download | SGP.32 §3.3.1 | The IPAd SHALL support profile download from an SM-DP+ server using an LPA activation code. | src/ipad.cpp::ipad_profile_download() ipad.h::ipad_dl_params_t::activation_code | IpadTest.DownloadProfileSync IpadTest.DownloadFailsWhenBackendFails IpadTest.DownloadNullActivationCodeReturnsError TV-DL-001 TV-DL-003 TV-DL-004 TC_WF_01_FullLifecycle BenchSuite.ProfileDownload | VERIFIED |
| REQ-021 | Profile Download | SGP.32 §3.3.2 | The IPAd SHALL support an optional confirmation code during profile download. | src/ipad.cpp::ipad_profile_download() — confirmation_code forwarded to addProfile ipad.h::ipad_dl_params_t::confirmation_code | TV-DL-006 (download with confirmation code) | VERIFIED |
| REQ-022 | Profile Download | SGP.32 §3.3.3 | The IPAd SHALL support asynchronous profile download with a caller-provided completion callback. | src/ipad.cpp::ipad_profile_download_async() ipad.h::ipad_callback_t | IpadTest.DownloadAsyncCallsCallback TV-DL-007 | VERIFIED |
| REQ-023 | Profile Download | SGP.32 §3.3.4 | The IPAd SHALL report download progress via IPAD_EVT_DOWNLOAD_PROGRESS events with a percentage value. | src/ipad.cpp::IpadProfileListener::onDownloadStatus() ipad.h::IPAD_EVT_DOWNLOAD_PROGRESS ipad.h::ipad_event_t::progress_pct | TC_WF_03_EventDrivenDownload BenchSuite.EventDeliveryLatency | VERIFIED |
| REQ-024 | Profile Download | SGP.32 §3.3.5 | The IPAd SHALL support automatic profile enablement immediately after a successful download. | src/ipad.cpp::ipad_profile_download() — auto_enable forwarded to addProfile ipad.h::ipad_dl_params_t::auto_enable | IpadTest.DownloadAndAutoEnable TV-DL-002 TC_WF_03_EventDrivenDownload | VERIFIED |
| REQ-030 | Profile Management | SGP.32 §3.4 | The IPAd SHALL list all profiles installed on the eUICC with their ICCID, state, provider, and nickname. | src/ipad.cpp::ipad_profile_list() ipad.h::ipad_profile_t | IpadTest.EmptyProfileList IpadTest.ListOneProfile IpadTest.ListMultipleProfiles IpadTest.ListFailsWhenBackendFails TV-LST-001..004 TC_WF_02_MultipleProfiles_Exclusivity TC_WF_05_ConcurrentListDuringDownload BenchSuite.ProfileListEmpty BenchSuite.ProfileListEight | VERIFIED |
| REQ-031 | Profile Management | SGP.32 §3.4 | The IPAd SHALL retrieve a single profile descriptor by ICCID. | src/ipad.cpp::ipad_profile_get() | IpadTest.EnableNonExistentProfileReturnsError (NOT_FOUND path) TC_WF_01_FullLifecycle (profile_get after delete) | VERIFIED |
| REQ-032 | Profile Management | SGP.32 §3.4.1 | The IPAd SHALL activate a profile (transition to ENABLED state via TelSDK setProfile). | src/ipad.cpp::ipad_profile_enable() → profile_set_state(enable=true) | IpadTest.EnableProfile IpadTest.EnableNonExistentProfileReturnsError TV-EN-001 TV-EN-002 TC_WF_01_FullLifecycle BenchSuite.ProfileEnableDisable | VERIFIED |
| REQ-033 | Profile Management | SGP.32 §3.4.2 | The IPAd SHALL enforce single-active-profile constraint: enabling one profile SHALL deactivate all others. | src/ipad.cpp — enforced by TelSDK setProfile; verified by unit + integration test | IpadTest.EnableProfileExclusivity TV-EN-003 TC_WF_02_MultipleProfiles_Exclusivity | VERIFIED |
| REQ-034 | Profile Management | SGP.32 §3.4.3 | The IPAd SHALL deactivate the active profile (transition to DISABLED state). | src/ipad.cpp::ipad_profile_disable() → profile_set_state(enable=false) | IpadTest.DisableProfile TV-DIS-001 TC_WF_01_FullLifecycle BenchSuite.ProfileEnableDisable | VERIFIED |
| REQ-035 | Profile Management | SGP.32 §3.4.4 | The IPAd SHALL permanently delete a profile from the eUICC via TelSDK deleteProfile. | src/ipad.cpp::ipad_profile_delete() | IpadTest.DeleteProfile IpadTest.DeleteNonExistentProfileReturnsError TV-DEL-001 TV-DEL-002 TC_WF_01_FullLifecycle | VERIFIED |
| REQ-036 | Profile Management | SGP.32 §3.4.5 | The IPAd SHALL allow updating a profile's human-readable nickname via TelSDK updateNickName. | src/ipad.cpp::ipad_profile_set_nickname() | IpadTest.UpdateNickname TV-NICK-001 TV-NICK-002 TC_WF_06_NicknamePersistence | VERIFIED |
| REQ-037 | Profile Management | SGP.32 §3.4 | The IPAd SHALL expose the current state of any installed profile on demand. | src/ipad.cpp::ipad_profile_get_state() | TC_WF_01_FullLifecycle (get_state assertions) TV-EN-001 (state_after) TV-DIS-001 (state_after) | VERIFIED |
| REQ-040 | Transport | SGP.32 §4.1 | The IPAd SHALL support TCP/IP as a transport protocol to the eIM. | ipad.h::IPAD_TRANSPORT_TCP ipad.h::ipad_config_t::transport | No dedicated functional test. Enum accepted at config init (IpadInitTest.SuccessfulInit); transport behaviour not exercised by the mock backend. | PARTIAL |
| REQ-041 | Transport | SGP.32 §4.2 | The IPAd SHALL support CoAP as a transport protocol to the eIM. | ipad.h::IPAD_TRANSPORT_COAP ipad.h::ipad_config_t::transport | No dedicated functional test. Enum accepted at config init only. | PARTIAL |
| REQ-042 | Transport | SGP.32 §4.3 | The IPAd SHALL support MQTT as a transport protocol to the eIM. | ipad.h::IPAD_TRANSPORT_MQTT ipad.h::ipad_config_t::transport | No dedicated functional test. Enum accepted at config init only. | PARTIAL |
| REQ-043 | Transport | SGP.32 §4.4 | The IPAd SHALL support SMS as a transport protocol to the eIM. | ipad.h::IPAD_TRANSPORT_SMS ipad.h::ipad_config_t::transport | No dedicated functional test. Enum accepted at config init only. | PARTIAL |
| REQ-050 | Security | SGP.32 §5.1 | The IPAd SHALL protect eIM communication with TLS 1.2 or higher (configurable minimum version). | ipad.h::ipad_tls_cfg_t::tls_version_min ipad.h::ipad_tls_cfg_t::ca_cert_path | No dedicated functional test. TLS config struct accepted at init; handshake not exercised by the mock backend. | PARTIAL |
| REQ-051 | Security | SGP.32 §5.2 | The IPAd SHALL support mutual TLS authentication via client certificate and private key. | ipad.h::ipad_tls_cfg_t::client_cert_path ipad.h::ipad_tls_cfg_t::client_key_path | No dedicated functional test. Config fields accepted at init only. | PARTIAL |
| REQ-052 | Security | SGP.32 §5.3 | The IPAd SHALL support configurable peer certificate verification. | ipad.h::ipad_tls_cfg_t::verify_peer | No dedicated functional test. Config field accepted at init only. | PARTIAL |
| REQ-060 | Event System | SGP.32 §6 | The IPAd SHALL provide an event subscription mechanism with bitmask-based type filtering. | src/ipad.cpp::ipad_event_subscribe() src/ipad.cpp::ipad_event_unsubscribe() ipad.h::ipad_event_type_t | IpadTest.EventSubscribeAndReceive IpadTest.UnsubscribeInvalidIdReturnsError TV-EVT-001 TV-EVT-002 TV-EVT-003 BenchSuite.EventSubscribeUnsubscribe | VERIFIED |
| REQ-061 | Event System | SGP.32 §6.1 | The IPAd SHALL emit events on every profile state transition: installed, enabled, disabled, deleted. | src/ipad.cpp::profile_set_state() — push_event(ENABLED/DISABLED) src/ipad.cpp::ipad_profile_delete() — push_event(DELETED) src/ipad.cpp::IpadProfileListener::onDownloadStatus() — push_event(INSTALLED) | IpadTest.EventSubscribeAndReceive TC_WF_03_EventDrivenDownload | VERIFIED |
| REQ-062 | Event System | SGP.32 §6.2 | The IPAd SHALL emit IPAD_EVT_EIM_CONNECTED and IPAD_EVT_EIM_DISCONNECTED events on TelSDK service status changes. | src/ipad.cpp::IpadProfileListener::serviceStatus() | IpadTest.ServiceStatusCallbacks | VERIFIED |
| REQ-063 | Event System | SGP.32 §6 | The IPAd SHALL support multiple concurrent independent event subscribers. | src/ipad.cpp::ipad_ctx_s::subscriptions (std::vector) src/ipad.cpp::dispatch_events() iterates all | Implemented and exercised indirectly (subscribe/unsubscribe tests), but no dedicated test asserts simultaneous delivery to two subscribers. | PARTIAL |
| REQ-064 | Event System | SGP.32 §6 | The IPAd SHALL support an explicit event pump for single-threaded environments. | src/ipad.cpp::ipad_event_pump() | IpadTest.EventPumpWithNoEvents IpadTest.EventPumpNullHandleReturnsError | VERIFIED |
| REQ-070 | Error Handling | SGP.32 §7 | The IPAd SHALL return structured numeric error codes for all API failures, with human-readable strings. | src/ipad.cpp::ipad_strerror() ipad.h — 16 defined status codes | IpadUtilTest.StrerrorKnownCodes IpadUtilTest.StrerrorUnknownCode TV-UTIL-001 IpadTest.TelsdkStatusMapping{Failed,InvalidParam,NotSupported} | VERIFIED |
| REQ-071 | Error Handling | SGP.32 §7.1 | The IPAd SHALL store a configurable retry count for transient communication failures. | src/ipad.cpp::ipad_init() — cfg.retry_count default=3 src/ipad.cpp::ipad_configure() | Recovery-after-failure is covered (TC_WF_04_RecoveryAfterFailure), but retry is performed at application level — no test asserts the stored retry_count drives internal re-attempts. | PARTIAL |
| REQ-072 | Error Handling | SGP.32 §7.2 | The IPAd SHALL apply a configurable operation timeout to every blocking TelSDK call. | src/ipad.cpp — cv.wait_for(timeout_ms) in ipad_get_eid, ipad_profile_list, ipad_profile_download, profile_set_state, ipad_profile_delete, ipad_profile_set_nickname | IpadInitTest.InitSubsystemReadyTimeout TV-INIT-003 (subsystem-ready timeout) TV-DL-005 (download timeout) | VERIFIED |
| REQ-080 | Diagnostics | SGP.32 §8 | The IPAd SHALL export a JSON-formatted diagnostic snapshot containing EID, profile count, and subsystem state. | src/ipad.cpp::ipad_diag_export_json() | IpadTest.DiagExportJsonIsValidJson TV-DIAG-001 TV-DIAG-002 TC_WF_07_DiagnosticAfterChanges BenchSuite.DiagExportJson | VERIFIED |
| REQ-081 | Diagnostics | SGP.32 §8.1 | The IPAd SHALL support five log levels — NONE, ERROR, WARN, INFO, DEBUG — with runtime filtering. | src/ipad.cpp::ipad_ctx_s::log() — level > cfg.log_level guard src/ipad.cpp::ipad_log_set_level() ipad.h::ipad_loglevel_t | IpadLogTest.NoneLevelProducesNoOutput IpadLogTest.TcpSinkSimpleLevelReceivesTimestampedLines IpadLogTest.TcpSinkDetailLevelReceivesDebugLines IpadTest.LogSetLevelNullHdlReturnsError BenchSuite.LogSetHandler | VERIFIED |
| REQ-082 | Diagnostics | SGP.32 §8.2 | The IPAd SHALL support a custom application-provided log handler with module and level context. | src/ipad.cpp::ipad_log_set_handler() ipad.h::ipad_log_cb_t | IpadTest.LogSetHandlerReceivesLogMessages IpadTest.LogSetHandlerNullHdlReturnsError IpadLogTest.DetachRestoresSilence IpadLogTest.AttachNullParams | VERIFIED |

# Test-to-Requirement Cross-Reference

The table below maps each test suite and individual test to the requirements it verifies.

## Unit Tests (ipad_unit_tests)

| Test Case | Requirements Verified |
| --- | --- |
| IpadInitTest.SuccessfulInit | REQ-001 |
| IpadInitTest.NullConfigReturnsInvalidParam | REQ-001 |
| IpadInitTest.NullHandleOutReturnsInvalidParam | REQ-001 |
| IpadInitTest.InitNullCardManagerFails | REQ-001 |
| IpadInitTest.InitSubsystemReadyTimeout | REQ-072 |
| IpadInitTest.DeinitNullHandleIsNoop | REQ-002 |
| IpadVersionTest.GetVersionPopulatesFields | REQ-004 |
| IpadVersionTest.GetVersionNullReturnsError | REQ-004 |
| IpadTest.GetEidReturnsExpectedValue | REQ-010 |
| IpadTest.GetEidFailsGracefully | REQ-010 |
| IpadTest.GetEidNullHandleReturnsError | REQ-010 |
| IpadTest.EmptyProfileList / ListOneProfile / ListMultipleProfiles | REQ-030 |
| IpadTest.ListFailsWhenBackendFails | REQ-030 |
| IpadTest.DownloadProfileSync | REQ-020 |
| IpadTest.DownloadFailsWhenBackendFails | REQ-020 |
| IpadTest.DownloadNullActivationCodeReturnsError | REQ-020 |
| IpadTest.DownloadAndAutoEnable | REQ-024 |
| IpadTest.DownloadAsyncCallsCallback | REQ-022 |
| IpadTest.DownloadWithUserConsent | REQ-020 |
| IpadTest.DownloadErrorAtCompletion / DownloadImmediateStatusFailure | REQ-020 REQ-070 |
| IpadTest.EnableProfile | REQ-032 |
| IpadTest.EnableNonExistentProfileReturnsError | REQ-031 REQ-032 |
| IpadTest.EnableProfileExclusivity | REQ-033 |
| IpadTest.DisableProfile | REQ-034 |
| IpadTest.DeleteProfile | REQ-035 |
| IpadTest.DeleteNonExistentProfileReturnsError | REQ-035 |
| IpadTest.UpdateNickname | REQ-036 |
| IpadTest.EventSubscribeAndReceive | REQ-060 REQ-061 |
| IpadTest.UnsubscribeInvalidIdReturnsError | REQ-060 |
| IpadTest.ServiceStatusCallbacks | REQ-062 |
| IpadTest.EventPumpWithNoEvents | REQ-064 |
| IpadTest.EventPumpNullHandleReturnsError | REQ-064 |
| IpadTest.DiagExportJsonIsValidJson | REQ-080 |
| IpadTest.ConfigureUpdatesSettings | REQ-003 |
| IpadTest.ConfigureNullParamsReturnsError | REQ-003 |
| IpadTest.SelfTestPasses | REQ-005 |
| IpadTest.LogSetHandlerReceivesLogMessages | REQ-082 |
| IpadTest.LogSetHandlerNullHdlReturnsError | REQ-082 |
| IpadTest.LogSetLevelNullHdlReturnsError | REQ-081 |
| IpadTest.TelsdkStatusMapping{Failed,InvalidParam,NotSupported} | REQ-070 |
| IpadUtilTest.StrerrorKnownCodes / StrerrorUnknownCode | REQ-070 |
| IpadUtilTest.IccidToStrRoundtrip / EidToStrRoundtrip | REQ-010 REQ-070 |

## Log Tests (ipad_log_tests)

| Test Case | Requirements Verified |
| --- | --- |
| IpadLogTest.NoneLevelProducesNoOutput | REQ-081 |
| IpadLogTest.TcpSinkSimpleLevelReceivesTimestampedLines | REQ-081 |
| IpadLogTest.TcpSinkDetailLevelReceivesDebugLines | REQ-081 |
| IpadLogTest.DetachRestoresSilence | REQ-082 |
| IpadLogTest.AttachNullParams / DetachNullHandle | REQ-082 |
| IpadLogTest.SinkTcpNullParams / SinkSerialNullParams | REQ-082 |
| IpadLogTest.TcpSinkConnectFailure / SerialOpenFailureOnBadDevice | REQ-082 |

## Integration Tests (ipad_integration_tests)

| Test Case | Requirements Verified |
| --- | --- |
| TC_WF_01_FullLifecycle | REQ-001 REQ-020 REQ-032 REQ-034 REQ-035 REQ-037 |
| TC_WF_02_MultipleProfiles_Exclusivity | REQ-030 REQ-033 |
| TC_WF_03_EventDrivenDownload | REQ-023 REQ-024 REQ-061 |
| TC_WF_04_RecoveryAfterFailure | REQ-071 |
| TC_WF_05_ConcurrentListDuringDownload | REQ-030 |
| TC_WF_06_NicknamePersistence | REQ-036 |
| TC_WF_07_DiagnosticAfterChanges | REQ-011 REQ-080 |

## SGP.32 Vector Runner (ipad_vector_runner)

| Test Case | Requirements Verified |
| --- | --- |
| TV-INIT-001..003 | REQ-001 REQ-072 |
| TV-EID-001..002 | REQ-010 |
| TV-LST-001..004 | REQ-030 |
| TV-DL-001..007 | REQ-020 REQ-021 REQ-022 REQ-024 REQ-072 |
| TV-EN-001..003 | REQ-032 REQ-033 REQ-037 |
| TV-DIS-001 | REQ-034 REQ-037 |
| TV-DEL-001..002 | REQ-035 |
| TV-NICK-001..002 | REQ-036 |
| TV-EVT-001..003 | REQ-060 |
| TV-DIAG-001..002 | REQ-080 |
| TV-UTIL-001 | REQ-070 |

## Benchmarks (ipad_bench)

| Test Case | Requirements Verified |
| --- | --- |
| BenchInit.InitDeinitCycle | REQ-001 REQ-002 |
| BenchSuite.GetEid | REQ-010 |
| BenchSuite.GetEuiccInfo | REQ-011 |
| BenchSuite.ProfileListEmpty / ProfileListEight | REQ-030 |
| BenchSuite.ProfileDownload | REQ-020 |
| BenchSuite.ProfileEnableDisable | REQ-032 REQ-034 |
| BenchSuite.EventSubscribeUnsubscribe | REQ-060 |
| BenchSuite.EventDeliveryLatency | REQ-023 REQ-061 |
| BenchSuite.DiagExportJson | REQ-080 |
| BenchSuite.LogSetHandler | REQ-081 REQ-082 |

IPAd Library v1.0.0 — Requirements Traceability Matrix
GSMA SGP.32 v1.0 | Qualcomm SA525 | © 2026